Environment Legislation:

Contents:

- Refrigeration

- Water Pollution

- Catering Waste

- Waste Brokers

- WEE

Refrigeration:

http://www.netregs.gov.uk/netregs/62479.aspx

- SCHEDULE 1 Table of Minimum Qualifications

| Column 1 | Column 2 | Column 3 |
| --- | --- | --- |
| Tasks | Equipment | Minimum Qualification |
| Servicing and maintenance of equipment. | Refrigeration, air conditioning and heat pump equipment which is stationary at all times when in operation. | (a) City & Guilds Certificate in Handling Refrigerants Scheme 2078(15). (b) Construction Industry Training Board Safe Handling of Refrigerants (J01)(16). (c) City & Guilds Level 2 Award in F Gas and ODS Regulations Scheme 2079-11: Category I or 2079-12: Category II. (d) Construction Industry Training Board Safe Handling of Refrigerants J11: Category I or J12: Category II. |
|  | Commercial and domestic refrigerators and freezers which are stationary at all times when in operation. |  |
|  | Portable refrigeration, air conditioning and heat pump equipment. |  |
| Dismantling of equipment. | Refrigeration, air conditioning and heat pump equipment— | (a) City & Guilds Certificate in Handling Refrigerants Scheme 2078. (b) Construction Industry Training Board Safe Handling of Refrigerants (J01). (c) City & Guilds Level 2 Award in F Gas and ODS Regulations Scheme 2079-11: Category I, 2079-12: Category II or 2079-13: Category III. (d) Construction Industry Training Board Safe Handling of Refrigerants J11: Category I, J12: Category II or J13: Category III. |
|  | (a) which is stationary at all times when in operation; and |  |
|  | (b) which can only be dismantled at the place at which the equipment is used. |  |
| Decommissioning of equipment. | Fire protection systems and fire extinguishers. | (a) British Fire Protection Systems Association Competence Certificate Course Class I. (b) Fire Industry Competence Certificate Course Class 1(17). |

Regulations 3(c) and 5(6)

Water Pollution:

http://publications.environment-agency.gov.uk/pdf/PMHO0501BFOX-e-e.pdf

- Good practice

Store all chemicals, including secondary refrigerants such as propylene glycol, in an area where spills will be contained. Ideally this will be within an impermeable bund, which will contain at least 110% of the volume of the largest tank or 25% of the total volume likely to be stored, whichever is greater. Remove any accumulated rainwater regularly, and if it is contaminated you may need to treat it as hazardous/special waste.

You should use competent specialist contractors when installing or decommissioning refrigeration plant. Supervise their work to ensure that leaks are prevented and that solutions containing the refrigerant do not enter watercourses or surface water drains.

Establish a regular maintenance and inspection schedule to ensure leaks do not occur.

Ammonia refrigerants are highly toxic. You should never allow them to enter watercourses or surface water drains.

If water contaminated with ammonia does enter a watercourse or surface water drain, report it immediately using the pollution hotline, tel: 0800 80 70 60.

If contaminated water enters a sewer, you should inform your statutory sewerage undertaker. This is normally your local water company in England and Wales, Scottish Water in Scotland and the Water Service in Northern Ireland.

Reproduced from: http://www.netregs.gov.uk/netregs/62479.aspx on

9th December 2009

Catering waste:

Catering waste is waste food from restaurants, catering facilities such as cafes and bars, and kitchens. It includes all food waste that is no longer intended for human consumption. Catering waste includes used cooking oil. You can no longer feed catering waste, including used cooking oil, to animals.

- What you must do

You can dispose of your catering waste to landfill, or to a composting or biogas plant. You must make sure that catering waste is collected and transported by a registered waste carrier who will dispose of it legally. If your catering waste does end up being used for animal feed then you could be prosecuted.

- Duty of care - your waste responsibilities

- Defra: Animal by-products - catering waste

The use of catering waste to produce compost and biogas is becoming more common as pressure on landfill and the associated costs increase. If you are passing your waste to a contractor for composting or production of biogas then they must pre-treat it and you must check that they hold the appropriate licenses and authorizations to do this. If you wish to produce compost or biogas yourself then you do not need to pre-treat it but you must meet a number of conditions.

- Composting catering waste

If your catering facilities are located on separate campuses, you may need to transport the catering waste to a central location for composting. You do not require a waste carriers license if you collect catering waste from your own catering facilities. You must, however, register as an exempt carrier with your environmental regulator.

- Contact your environmental regulator

If you transport your own business waste, you do not need to register as a waste carrier, unless you are transporting building or demolition waste.

Waste carriers, brokers and dealers:

You can apply the compost that you produce to your own grounds, even if your institution has a number of separate campuses.

You must not dispose of used cooking oil in sewers or drains. Cooking oil can block the sewer and can create problems with vermin. Used cooking oil can cause serious problems if it reaches a wastewater treatment plant.

You can no longer send used cooking oil for use in the production of animal feed. You should collect used oil for treatment and make sure that you don’t dispose of it along with other catering waste. A number of businesses will collect waste cooking oil at no charge to the customer. Much of this is turned into bio-diesel.

You can treat your waste cooking oil to produce bio-diesel for use in your own vehicles. You may require an environmental permit (England and Wales) or a pollution prevention and control (PPC) permit (Northern Ireland and Scotland) to reprocess waste catering oil into bio-fuels. This will depend on the type of process you carry out and the volume of your operation.

As a general guide, in England, Northern Ireland and Wales, if you produce more than 5,000 liters of oil, your business is likely to require a permit.

In Scotland, if you carry out transesterification of waste catering oils up to 200 tonnes per year on a non commercial scale, you do not require a PPC permit. You should contact your environmental regulator to check whether you qualify for this exemption.

- Contact your environmental regulator

- Environment Agency: Production of biodiesel - what you must do

- SEPA: Guidance for small scale biodiesel manufacture (Adobe PDF - 26KB)

Never dispose of cooking oil in waste oil banks that are intended for engine oil. This makes the contents of the bank unusable.

You must store waste cooking oil safely and securely according to the same regulations which govern the storage of any oil.

- Oil storage

- Good practice

Carry out a waste audit to identify the main types of waste from your catering facilities. This will allow you to identify the materials that contribute most to your catering waste. You can then take steps to reduce the waste you produce.

Discuss with your waste contractor the opportunities for recycling common materials. You may be able to arrange separate collections of glass, cans, plastic bottles, cardboard and mixed waste.

http://www.netregs.gov.uk/netregs/62521.aspx

This environmental topic will help you work out if you are a waste carrier, broker or dealer. It will tell you if you need to register with your environmental regulator, or if you qualify for an exemption from registering.

If you want to transport your own or somebody else’s waste, see our guidance for waste carriers.

Waste carriers

If you want to deal in waste or arrange for someone else’s waste to be disposed of or recovered, see our guidance for waste brokers and dealers.

Waste brokers and dealers

If you want to transport, deal in or arrange the disposal or recovery of any controlled waste, you may need to register with your environmental regulator as a waste carrier and/or waste broker.

Controlled waste includes commercial, industrial and household waste, as well as hazardous/special waste. Examples of non-controlled wastes include:

- radioactive waste

- explosive waste.

For information on how to deal with radioactive waste see our guidance on radioactive substances and wastes.

The Health and Safety Executive (HSE) has produced guidance on disposing of explosive waste

WEE:

The Waste Electrical and Electronic Equipment (WEEE) Regulations apply to electrical and electronic equipment (EEE) in the categories listed below with a voltage of up to 1000 volts for alternating current or up to 1500 volts for direct current.

You will need to comply with the WEEE Regulations if you generate, handle or dispose of waste that falls under one of ten categories of WEEE:

- Large household appliances

- Small household appliances

- IT and telecommunications equipment

- Consumer equipment

- Lighting equipment

- Electrical and electronic tools

- Toys, leisure and sports equipment

- Medical devices

- Monitoring and control equipment

- Automatic dispensers.